SPCAA Head Start Transportation (October 2007)

Procedures to Winterize Vehicles

Name of Driver Completing this Form 						Date:

Signature of Person Performing the Winterize checks 			Date:

Drivers are responsible for Winterizing vehicles assigned to them during the weeks of October 22 – November 9, 2007. Items with a  can be taken to an approved mechanic to complete. All drivers must FAX return a completed copy of this form to Robert Whitfield by close of business on 11/09/07 for the vehicle assigned to them. The original of this form should be kept in the silver tray in the vehicle.

A. Cooling System Check – Do this before vehicle has been started for the day

NEVER REMOVE RADIATOR CAP WHEN ENGINE IS HOT

- Remove Radiator Cap – Fluids should be a clear green color.

If water is brown or dirty looking, the vehicle may need a radiator flush, call Robert Whitfield if fluid is not a clear green color.

- Use an antifreeze tester – temp should be -20 degrees. If temperature is above (less than) -20, call Robert Whitfield (negative or -34 is great) (Note a vehicle with -10 is less than -20; a vehicle with -30 is more than -20)

- Check the overflow bottle for level – if overflow bottle is low add antifreeze. Never add just water to the radiator/overflow system during the winter. Write down on the pre-trip the amount of antifreeze you added.

- If the system is very low, check for leaks or wet spot around radiator, hoses, or engine. If system is very low call Robert Whitfield

B. Windshield wipers must be in good condition. If wipers need to be replaced, call 	Robert Whitfield for approval to get repairs.

C. Vehicles will need to start cold and run hot – this puts more pressure on hoses and 		belts.

1.	Double check all belts and hoses for worn or frayed spots.

2.	Check clamps on hoses.

3.	Check for leak stains on hoses – stains are hints of small leaks.

D. Battery – If it has been hard to start the vehicle in moderate temperature, check 	with Robert Whitfield to see about a replacement. Cold weather reduces the operating 	efficiency of the battery. NEVER add water to a battery.

E. Roads with snow or rain will splash on the windows and hinder visibility. Check 	windshield wiper solvent – container must remain full at the end of each trip. Be sure 	Window cleaner/paper towels are in the vehicle at all times. Window scraper (with a 	vehicle number) MUST be in the vehicle at all times. Never use ice scraper on mirrors 	or cat eyes (curved mirrors) – use deicer to defrost these ~ or ~ cover mirrors 	overnight with plastic bag.